- Final Cruise Instructions

FOCI

- NOAA Ship MILLER FREEMAN, MF-03-12

- September 23 – October 6, 2003

- Chief Scientist: Carol L. DeWitt, NOAA/PMEL

- Final Cruise Instructions

  - Cruise Title – Fisheries-Oceanography Coordinated Investigations (FOCI)

  - Cruise Numbers

    - Cruise Number – MF-03-12

    - FOCI Number – 8MF03

  - Cruise Dates:

    - Departure – Tuesday, September 23, 2003, at 09:00 ADT, from Kodiak, Alaska.

    - “Touch-n-Go” Arrival – Friday, October 3, 2003, at 07:30 ADT, in Dutch Harbor, Alaska.  All scientists, except Bill Floering and Earl Roskie, will depart the ship.  Bill Floering and Earl Roskie will remain on the ship for the dragging operations at the FATE mooring site and the transit back to Seattle.

    - “Touch-n-Go” Departure – Friday, October 3, 2003, at 10:30 ADT, in Dutch Harbor, Alaska.

    - Arrival – Saturday, October 11, 2003, in Seattle, Washington.

  - Operating Area – Gulf of Alaska and Bering Sea.

- Cruise Overview

  - Cruise Objectives – The primary objective of this cruise will be the recovery and deployment of moorings in the Gulf of Alaska and Bering Sea.  The following mooring operations will be conducted on this cruise:

| OPERATIONS | SITE | LATITUDE | LONGITUDE |
| --- | --- | --- | --- |
| Recover/Deploy | Mooring 2 | 56° 51.613' N | 164° 03.639' W |
| Recover/Deploy | Mooring 4 | 57° 51.181' N | 168° 52.182' W |
| Recover/Deploy | Amukta Pass | 52° 22.971' N | 172° 07.199' W |
| Recover/Deploy | Samalga Pass | 53° 01.301' N | 168° 59.915' W |
| Recover/Deploy | Shelikof Strait – Line 8 | 57° 41.094' N | 155° 12.205' W |
| Recover/Deploy | Chiniak Bay | 57° 43.346' N | 152° 17.661' W |
| Drag | FATE | 58° 15.612' N | 147° 41.214' W |

  - Applicability – These instructions, with FOCI Standard Operating Instructions for NOAA Ship MILLER FREEMAN, dated April 8, 2002, present complete information for this cruise.

  - Participating Organizations

NOAA – Pacific Marine Environmental Laboratory (PMEL)

7600 Sand Point Way N.E.

Seattle, Washington 98115-6439

NOAA – Alaska Fisheries Science Center (AFSC)

7600 Sand Point Way N.E.

Seattle, Washington 98115-0070

University of Alaska – Fairbanks (UAF)

Institute of Marine Science

200 O’Neill

Fairbanks, Alaska 99775-1080

  - Personnel

    - Chief Scientist

| Name | Gender | Affiliation | E-mail Address |
| --- | --- | --- | --- |
| Carol L. DeWitt | Female | PMEL | Carol.DeWitt@noaa.gov |
| (206) 526-6522 |  |  |  |

    - Participating Scientists

| Name | Gender | Affiliation | E-mail Address |
| --- | --- | --- | --- |
| Carol L. DeWitt | Female | PMEL | Carol.DeWitt@noaa.gov |
| William J. Floering | Male | PMEL | William.Floering@noaa.gov |
| Earl Roskie | Male | PMEL | Earl.Roskie@noaa.gov |
| Stephen A. Smith | Male | PMEL | Stephen.A.Smith@noaa.gov |
| Sarah Thornton | Female | UAF | sarahjt@imsuaf.edu |
| Susan Henrichs | Female | UAF | henrichs@ims.uaf.edu |

  - Administration

    - Ship Operations

- Marine Operations Center, Pacific

1801 Fairview Avenue East

Seattle, Washington 98102-3767

Telephone: (206) 553-4548

Fax: (206) 553-1109

- Commander Michele G. Bullock, NOAA

Chief, Operations Division, Pacific (MOP1)

Telephone: (206) 553-8705

Cellular: (206) 390-7527

- E-mail: Michelle.Bullock@noaa.gov

- Larry W. Mordock

Deputy Chief, Operations Division (MOP1x1)

Telephone – Work: (206) 553-4764

Home: (206) 365-3567

Cellular: (206) 465-9316

- E-mail:  Larry.Mordock@noaa.gov

- Scientific Operations

Dr. Phyllis J. Stabeno, NOAA/PMEL	Dr. Jeffrey M. Napp, NOAA/AFSC

Telephone: (206) 526-6453			Telephone: (206) 526-4148

E-mail: Phyllis.Stabeno@noaa.gov		E-mail: Jeff.Napp@noaa.gov

- Operations

  - Data To Be Collected

    - Scientific Computer System (SCS) – The ship's SCS shall operate throughout the cruise, acquiring and logging data from navigation, meteorological, oceanographic, and fisheries sensors.  See FOCI Standard Operating Instructions for NOAA Ship MILLER FREEMAN (SOI 5.2) for specific requirements.

  - Staging Plan – The vessel will be equipped with all necessary equipment prior to the ship’s departure from Seattle, Washington, on Tuesday, September 2, 2003.

  - De-staging Plan – All equipment loaded onto the vessel for, or collected during the cruise, will be offloaded in Seattle, Washington, between October 11 and October 29, 2003.

  - Cruise Plan – The ship will depart Kodiak, Alaska, on Tuesday, September 23, 2003, and steam directly to the first mooring at Chiniak Bay.

- Chiniak Bay – One mooring will be recovered and redeployed at Chiniak Bay.  Conductivity, Temperature, and Depth (CTD) profiles will be conducted before and after the mooring operations.

- Shelikof Strait/Line 8 – Three moorings will be recovered and redeployed along Line 8 in Shelikof Strait.  CTDs will be conducted before and after mooring operations at each site.

- FOCI Bering Sea Site 2 – Prior to mooring operations, a calibration CTD (with nutrient and chlorophyll samples), Marine Assessment Monitoring and Prediction (MARMAP) Bongo tow, and triplicate California Cooperative Oceanic Fisheries Investigation (CalCOFI) Vertical Egg Tow (CalVET) will be completed.  Mooring operations will consist of recovering one surface and two subsurface moorings and deploying two subsurface moorings.  After the completion of all mooring operations, a CTD will be completed.  A CTD, with nutrient and chlorophyll samples, and a MARMAP Bongo tow will be completed at the four stations surrounding Site 2.

- FOCI Bering Sea Site 4 – The ship will transit from FOCI Bering Sea Site 2 to FOCI Bering Sea Site 4.  Prior to mooring operations, a calibration CTD, with nutrient and chlorophyll samples, a MARMAP Bongo tow and triplicate CalVET tows will be completed.  Mooring operations will consist of recovering and redeploying one subsurface mooring.  After the completion of all mooring operations, a CTD will be completed.  A CTD, with nutrient and chlorophyll samples, and a MARMAP Bongo tow will be completed at the four stations surrounding Site 4.

- Amukta Pass – Four moorings will be recovered and redeployed at Amukta Pass.  Due to time constraints, a CTD transect, consisting of four CTDs across the pass, will occur only once, either prior to mooring recoveries or after mooring deployments.

- Samalga Pass – Five moorings will be recovered and redeployed at Samalga Pass.  CTDs at the mooring sites will be completed as time allows.

- FATE – As time allows, after the touch-and-go in Dutch Harbor, Alaska, dragging operations will be conducted for the FATE mooring.

  - Station Locations – Cruise itinerary and mooring sites can be found in Sections 9.2 Cruise MF-03-12 Station Locations and 9.3 Cruise MF-03-12 Mooring Sites.

  - Station Operations – The following are operations to be conducted on this cruise.  The procedures for these operations are listed in the FOCI Standard Operating Instructions for NOAA Ship MILLER FREEMAN (SOI).  Operations not addressed in the SOI and changes to standard procedures are addressed below:

- CTD/Water Sample Operations (SOI 3.2.1),

- MARMAP Bongo Tows (SOI 3.2.2),

- CalVET Net Tows (SOI 3.2.6), and

- Chlorophyll Sampling Operations (SOI 3.2.10).

  - Underway Operations – The following are underway operations to be conducted on this cruise.  The procedures for these operations are listed in the FOCI Standard Operating Instructions for NOAA Ship MILLER FREEMAN (SOI).  Operations not addressed in the SOI and changes to standard procedures are addressed below:

- Acoustic Doppler Current Profiler (ADCP) Operations (SOI 3.2.13),

- Radiometer Operations (SOI 3.2.14),

- Scientific Computer System (SCS) data acquisition (SOI 5.2), and

- Thermosalinograph monitoring (SOI 5.3).

  - Small Boat Operations – Weather permitting, the small boat may be needed for mooring operations.

- Facilities

  - Equipment and Capabilities Provided by Ship

- Oceanographic winch with slip rings and 3-conductor cable terminated for CTD,

- Manual wire-angle indicator,

- Oceanographic winch with slip rings and 3-conductor cable terminated for the SBE SEACAT, for net tow operations,

- Sea-Bird Electronics’ SBE 911plus CTD system with stand, each CTD system should include underwater CTD, weights, and pinger.  There should be one deck unit for the two systems,

- 10 liter Niskin sampling bottles for use with rosette (10 plus 4 spares),

- AUTOSAL salinometer, for CTD field corrections,

- Sea-Bird Electronics’ SBE-19 SEACAT system,

- Wire speed indicators and readout for quarterdeck, Rowe, and Marco winches,

- For meteorological observations: 2 anemometers (one R. M. Young system interfaced to the SCS), calibrated air thermometer (wet-and dry-bulb) and a calibrated barometer and/or barograph,

- Freezer space for storage of biological and chemical samples,

- RD Instruments’ ADCP written to Iomega Zip drive,

- Use of Pentium PC in DataPlot for data analysis,

- Scientific Computer System (SCS),

- Removable stern platform (in place),

- Laboratory space with exhaust hood, sink, lab tables and storage space,

- Sea-water hoses and nozzles to wash nets (quarterdeck and aft deck),

- Adequate deck lighting for night-time operations,

- Navigational equipment including GPS and radar,

- Safety harnesses for working on quarterdeck and fantail, and

- Ship’s crane(s) used for loading and/or deploying.

  - Equipment and Capabilities Provided by Scientists

- Sea-Bird Electronics’ SBE 911plus CTD system with dual sensors (for backup),

- Photosynthetically Active Radiation (PAR) and Fluorometer to be mounted on CTD,

- CTD stand modified for attachment of fluorometer,

- CTD carousel sampler,

- (12) 5 liter sample bottles,

- 60-cm MARMAP Bongo sampling arrays,

- 20-cm MARMAP Bongo sampling arrays,

- Spare wire angle indicator,

- 16 subsurface moorings,

- Benthos acoustic release deck-set and transducer,

- EdgeTech acoustic release deck-set and transducer,

- 16 railroad wheel sets to be used as anchors, and

- Miscellaneous scientific sampling and processing equipment

- Disposition of Data and Reports

  - The following data products will be included in the cruise data package:

- NOAA Form 77-13d – Deck Log – Weather Observation Sheets,

- Electronic Marine Operations Abstracts,

- SCS backup – recordable compact diskette (CD),

- Calibration Sheets for all ship's instruments used,

- CTD Cast Information/Rosette Log,

- Autosalinometer Logs,

- ADCP Log Sheets,

- ADCP Iomega Zip and/or recordable compact diskette (CD), and

- Ultra-cold Freezer Temperature Daily Log (SOI 5.4).

  - Pre- and Post-cruise Meetings – Cruise meetings may be held in accordance with FOCI Standard Operating Instructions for NOAA Ship MILLER FREEMAN (SOI 5.5).

- Additional Projects

  - Definition – Ancillary and piggyback projects are secondary to the objectives of the cruise and should be treated as additional investigations.  The difference between the two types of secondary projects is that an ancillary project does not have representation aboard and is accomplished by the ship's force.

  - Ancillary Projects – Any ancillary work done during this project will be accomplished with the concurrence of the Chief Scientist and on a not-to-interfere basis with the programs described in these instructions and in accordance with the NOAA Fleet Standing Ancillary Instructions.

  - Piggyback Projects – None.

- Hazardous Materials

  - HAZMAT Inventory – See Section 9.1 Cruise MF-03-12 HAZMAT Inventory.

  - Material Safety Data Sheet (MSDS) – All MSDSs can be found on the OERD HAZMAT Emergency Guidelines – MSDS compact diskette dated January 15, 2003, supplied to the ship.

- Miscellaneous

  - Communications – Specific information on how to contact the NOAA Ship MILLER FREEMAN and all other fleet vessels can be found at:

http://www.pmc.noaa.gov/phone.htm

  - Important Telephone and Facsimile Numbers and E-mail Addresses

    - Pacific Marine Environmental Laboratory (PMEL)

FOCI – Ocean Environmental Research Division (OERD2)

- (206) 526-4700 (voice)

- (206) 526-6485 (fax)

Administration:

- (206) 526-6810 (voice)

- (206) 526-6815 (fax)

    - Alaska Fisheries Science Center (AFSC)

FOCI – Resource Assessment and Conservation Engineering (RACE):

- (206) 526-4171 (voice)

- (206) 526-6723 (fax)

- E-Mail: FirstName.LastName@noaa.gov

    - NOAA Ship MILLER FREEMAN – Telephone methods listed in order of increasing expense:

Homeport – Seattle, Washington:

- (206) 553-4589

- (206) 553-4581

- (206) 553-8344

- United States Coast Guard – Kodiak, Alaska

- (907) 487-9752

- (907) 487-9753

- (907) 487-4397

- (907) 487-4398

Cellular:

- (206) 660-7167

- INMARSAT Mini-M

- 011-872-761-267-346 (voice/PBX)

- 011-872-761-267-347 (voice)

- INMARSAT B

- 011-872-330-394-120 (voice)

- 011-872-330-394-121 (fax)

E-Mail: NOAA.Ship.Miller.Freeman@noaa.gov (mention the person’s name in SUBJECT field)

    - Marine Operations Center, Pacific (MOP)

Operations Division (MOP1):

- (206) 553-4548 (voice)

- (206) 553-1109 (facsimile)

- E-Mail: FirstName.LastName@noaa.gov

- E-Mail to Radio Room: Radio.Room@noaa.gov

- Appendices

  - Cruise MF-03-12 HAZMAT Inventory

| Chemical | CAS Number | Respondee | Organization | Quantity | H | F | R | Storage Color Code | Hazard Class | Packing Group Number | UN | Reportable Quantity | Response Indices |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Battery, Lithium | mix | DeWitt | PMEL |  | 2 | 2 | 3 | General | 9 | II | 3090 | 35-kg | None |
| Hydrochloric Acid | 7647-01-0 | Thornton | UAF | 500-ml | 3 | 0 | 2 | Corrosive | 8 | II | 1789 | 5,000-lbs | 1 |
| Tributyltin Oxide | 56-35-9 | DeWitt | PMEL |  | 3 | 1 | 0 | General | Not reg. |  |  |  |  |

Spill Response 1: Ventilate area of leak or spill. Wear appropriate personal protective equipment. Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Neutralize with alkaline material (soda ash, lime), then absorb with an inert material (e.g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as sawdust. Do not flush to sewer! U.S. Regulations (CERCLA) requires reporting spills and releases to soil, water, and air in excess of reportable quantities. The toll free number for the U.S. Coast Guard National Response Center is (800) 424-8802.

  - Cruise MF-03-12 Station Locations

| Activity | Latitude |  |  | Longitude |  |  | Dist. (nm) | Spd (kts) | Trans (hrs) | Approx Bott Depth (m) | On Sta (hrs) | Arrive (Local) Date / Time | Depart (Local) Date/Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Depart Kodiak, Alaska | 57° | 45.000' | N | 152° | 29.600' | W |  |  |  |  |  |  | 23-Sep 09:00 |
| CTD at 03CB-1A | 57° | 43.346' | N | 152° | 17.661' | W | 6.6 | 11 | 0.6 | 184 | 0.5 | 23-Sep 09:35 | 23-Sep 10:08 |
| Recover 03CB-1A | 57° | 43.346' | N | 152° | 17.661' | W | 0.0 | 12 | 0.0 | 184 | 0.7 | 23-Sep 10:08 | 23-Sep 10:50 |
| Deploy 03CB-1B | 57° | 43.346' | N | 152° | 17.661' | W | 0.0 | 13 | 0.0 | 184 | 0.7 | 23-Sep 10:50 | 23-Sep 11:32 |
| CTD at 03CB-1B | 57° | 43.346' | N | 152° | 17.661' | W | 0.0 | 14 | 0.0 | 184 | 0.5 | 23-Sep 11:32 | 23-Sep 12:04 |
| CTD at 03SSP-1A | 57° | 41.094' | N | 155° | 12.205' | W | 103.8 | 10 | 10.4 | 292 | 0.6 | 24-Sep 06:28 | 24-Sep 07:03 |
| CTD at 03SSP-2A | 57° | 37.126' | N | 155° | 04.489' | W | 5.7 | 10 | 0.6 | 249 | 0.5 | 24-Sep 07:37 | 24-Sep 08:10 |
| CTD at 02SSP-3A | 57° | 29.016' | N | 154° | 48.440' | W | 11.8 | 10 | 1.2 | 198 | 0.6 | 24-Sep 09:21 | 24-Sep 09:54 |
| Recover 03SSP-3A | 57° | 29.016' | N | 154° | 48.440' | W | 0.0 | 10 | 0.0 | 198 | 1.0 | 24-Sep 09:54 | 24-Sep 10:54 |
| Deploy 03SSP-3B | 57° | 29.016' | N | 154° | 48.440' | W | 0.0 | 10 | 0.0 | 198 | 2.0 | 24-Sep 10:54 | 24-Sep 12:54 |
| CTD at 03SSP-3B | 57° | 29.016' | N | 154° | 48.440' | W | 0.0 | 10 | 0.0 | 198 | 0.6 | 24-Sep 12:54 | 24-Sep 13:27 |
| Recover 03SSP-2A | 57° | 37.126' | N | 155° | 04.489' | W | 11.8 | 10 | 1.2 | 249 | 1.0 | 24-Sep 14:38 | 24-Sep 15:38 |
| Deploy 03SSP-2B | 57° | 37.126' | N | 155° | 04.489' | W | 0.0 | 10 | 0.0 | 249 | 2.0 | 24-Sep 15:38 | 24-Sep 17:38 |
| CTD at 03SSP-2B | 57° | 37.126' | N | 155° | 04.489' | W | 0.0 | 10 | 0.0 | 249 | 0.5 | 24-Sep 17:38 | 24-Sep 18:11 |
| Recover 03SSP-1A | 57° | 41.094' | N | 155° | 12.205' | W | 5.7 | 10 | 0.6 | 292 | 1.0 | 24-Sep 18:45 | 24-Sep 19:45 |
| Deploy 03SSP-1B | 57° | 41.094' | N | 155° | 12.205' | W | 0.0 | 10 | 0.0 | 292 | 2.0 | 24-Sep 19:45 | 24-Sep 21:45 |
| CTD at 03SSP-1B | 57° | 41.094' | N | 155° | 12.205' | W | 0.0 | 10 | 0.0 | 292 | 0.6 | 24-Sep 21:45 | 24-Sep 22:20 |
| CTD - site 2/south | 56° | 40.000' | N | 163° | 52.000' | W | 132.1 | 10 | 13.2 | 65 | 0.4 | 27-Sep 07:04 | 27-Sep 07:28 |
| 20/60 bongo - site 2/south | 56° | 40.000' | N | 163° | 52.000' | W | 0.0 | 10 | 0.0 | 65 | 0.3 | 27-Sep 07:28 | 27-Sep 07:44 |
| CTD - site 2/middle | 56° | 51.969' | N | 164° | 03.066' | W | 13.4 | 10 | 1.3 | 198 | 0.6 | 27-Sep 09:05 | 27-Sep 09:38 |
| 20/60 Bongo - site 2/middle | 56° | 51.969' | N | 164° | 03.066' | W | 0.0 | 10 | 0.0 | 198 | 0.4 | 27-Sep 09:38 | 27-Sep 10:04 |
| CalVET (triplicate tows) (site 2/middle) | 56° | 51.969' | N | 164° | 03.066' | W | 0.0 | 10 | 0.0 | 198 | 0.5 | 27-Sep 10:04 | 27-Sep 10:34 |
| Recover 03BSP-2B | 56° | 51.613' | N | 164° | 03.639' | W | 0.5 | 10 | 0.0 | 73 | 1.0 | 27-Sep 10:37 | 27-Sep 11:37 |
| Recover 03BSM-2A | 56° | 51.969' | N | 164° | 03.066' | W | 0.5 | 10 | 0.0 | 73 | 1.0 | 27-Sep 11:40 | 27-Sep 12:40 |
| Recover 03BSST-2A | 56° | 52.000' | N | 164° | 02.516' | W | 0.3 | 10 | 0.0 | 73 | 1.0 | 27-Sep 12:42 | 27-Sep 13:42 |
| Deploy 03BS-2B | 56° | 51.969' | N | 164° | 03.066' | W | 0.3 | 10 | 0.0 | 73 | 2.0 | 27-Sep 13:44 | 27-Sep 15:44 |
| Deploy 03BSP-2C | 56° | 51.613' | N | 164° | 03.639' | W | 0.5 | 10 | 0.0 | 73 | 2.0 | 27-Sep 15:47 | 27-Sep 17:47 |
| CTD - Middle Domain (M2) | 56° | 51.969' | N | 164° | 03.066' | W | 0.5 | 10 | 0.0 | 73 | 0.4 | 27-Sep 17:49 | 27-Sep 18:14 |
| CTD - site 2/east | 56° | 56.500' | N | 163° | 50.010' | W | 8.4 | 10 | 0.8 | 65 | 0.4 | 27-Sep 19:05 | 27-Sep 19:29 |
| 20/60 bongo - site 2/east | 56° | 56.500' | N | 163° | 50.000' | W | 0.0 | 10 | 0.0 | 65 | 0.3 | 27-Sep 19:29 | 27-Sep 19:45 |
| CTD - site 2/north | 57° | 01.000' | N | 164° | 13.000' | W | 13.3 | 10 | 1.3 | 65 | 0.4 | 27-Sep 21:05 | 27-Sep 21:29 |
| 20/60 bongo - site 2/north | 57° | 01.000' | N | 164° | 13.001' | W | 0.0 | 10 | 0.0 | 65 | 0.3 | 27-Sep 21:29 | 27-Sep 21:45 |
| CTD - site 2/west | 56° | 46.000' | N | 164° | 20.000' | W | 15.5 | 10 | 1.5 | 75 | 0.4 | 27-Sep 23:18 | 27-Sep 23:43 |
| 20/60 bongo - site 2/west | 56° | 46.000' | N | 164° | 20.000' | W | 0.0 | 10 | 0.0 | 75 | 0.3 | 27-Sep 23:43 | 27-Sep 23:58 |
| CTD /Chlorophyll/Nutrients (site 4) | 57° | 51.181' | N | 168° | 52.182' | W | 160.8 | 10 | 16.1 | 72 | 0.4 | 28-Sep 16:03 | 28-Sep 16:27 |
| 20/60 Bongo (site 4) | 57° | 51.181' | N | 168° | 52.182' | W | 0.0 | 10 | 0.0 | 72 | 0.3 | 28-Sep 16:27 | 28-Sep 16:43 |
| CalVET (triplicate tows) (site 4) | 57° | 51.181' | N | 168° | 52.182' | W | 0.0 | 10 | 0.0 | 72 | 0.5 | 28-Sep 16:43 | 28-Sep 17:13 |
| Recover 03BS-4A | 57° | 51.181' | N | 168° | 52.182' | W | 0.0 | 10 | 0.0 | 72 | 1.0 | 28-Sep 17:13 | 28-Sep 18:13 |
| Deploy 03BS-4B | 57° | 51.181' | N | 168° | 52.182' | W | 0.0 | 10 | 0.0 | 72 | 2.0 | 28-Sep 18:13 | 28-Sep 20:13 |
| CTD at site 4 | 57° | 51.181' | N | 168° | 52.182' | W | 0.0 | 10 | 0.0 | 72 | 0.4 | 28-Sep 20:13 | 28-Sep 20:38 |
| CTD /Chlorophyll/Nutrients (site 4 - east) | 57° | 46.000' | N | 168° | 28.000' | W | 13.9 | 10 | 1.4 | 65 | 0.4 | 28-Sep 22:01 | 28-Sep 22:26 |
| 20/60 Bongo (site 4 - east) | 57° | 46.000' | N | 168° | 28.000' | W | 0.0 | 10 | 0.0 | 65 | 0.4 | 28-Sep 22:26 | 28-Sep 22:52 |
| CTD /Chlorophyll/Nutrients (site 4 - north) | 58° | 04.000' | N | 168° | 43.800' | W | 19.9 | 10 | 2.0 | 65 | 0.4 | 29-Sep 00:51 | 29-Sep 01:16 |
| 20/60 Bongo (site 4 - north) | 58° | 04.000' | N | 168° | 43.800' | W | 0.0 | 10 | 0.0 | 65 | 0.3 | 29-Sep 01:16 | 29-Sep 01:31 |
| CTD /Chlorophyll/Nutrients (site 4 - west) | 57° | 55.600' | N | 169° | 19.300' | W | 20.6 | 10 | 2.1 | 65 | 0.4 | 29-Sep 03:35 | 29-Sep 03:59 |
| 20/60 Bongo (site 4 - west) | 57° | 55.600' | N | 169° | 19.300' | W | 0.0 | 10 | 0.0 | 65 | 0.3 | 29-Sep 03:59 | 29-Sep 04:14 |
| CTD /Chlorophyll/Nutrients (site 4- south) | 57° | 39.200' | N | 169° | 01.200' | W | 19.0 | 10 | 1.9 | 60 | 0.4 | 29-Sep 06:08 | 29-Sep 06:32 |
| 20/60 Bongo (site 4 - south) | 57° | 39.200' | N | 169° | 01.200' | W | 0.0 | 10 | 0.0 | 60 | 0.3 | 29-Sep 06:32 | 29-Sep 06:48 |
| CTD at 03SG-3A | 53° | 01.301' | N | 168° | 59.915' | W | 277.9 | 11 | 25.3 | 104 | 0.4 | 30-Sep 08:04 | 30-Sep 08:28 |
| Recover 03SG-3A | 53° | 01.301' | N | 168° | 59.915' | W | 0.0 | 10 | 0.0 | 104 | 0.7 | 30-Sep 08:28 | 30-Sep 09:10 |
| Deploy 03SG-3B | 53° | 01.301' | N | 168° | 59.915' | W | 0.0 | 10 | 0.0 | 104 | 0.7 | 30-Sep 09:10 | 30-Sep 09:52 |
| CTD at 03SG-3B | 53° | 01.301' | N | 168° | 59.915' | W | 0.0 | 10 | 0.0 | 104 | 0.4 | 30-Sep 09:52 | 30-Sep 10:16 |
| CTD at 03SG-2A | 53° | 01.728' | N | 169° | 32.074' | W | 19.3 | 11.5 | 1.7 | 119 | 0.4 | 30-Sep 11:57 | 30-Sep 12:21 |
| Recover 03SG-2A | 53° | 01.728' | N | 169° | 32.074' | W | 0.0 | 10 | 0.0 | 119 | 0.7 | 30-Sep 12:21 | 30-Sep 13:03 |
| Deploy 03SG-2B | 53° | 01.728' | N | 169° | 32.074' | W | 0.0 | 10 | 0.0 | 119 | 0.7 | 30-Sep 13:03 | 30-Sep 13:45 |
| CTD at 03SG-2B | 53° | 01.728' | N | 169° | 32.074' | W | 0.0 | 10 | 0.0 | 119 | 0.4 | 30-Sep 13:45 | 30-Sep 14:09 |
| CTD/nutrients at 03SGP-1A | 52° | 50.776' | N | 169° | 27.271' | W | 11.3 | 11.5 | 1.0 | 229 | 0.4 | 30-Sep 15:08 | 30-Sep 15:32 |
| Recover 03SGP-1A | 52° | 50.776' | N | 169° | 27.271' | W | 0.0 | 10 | 0.0 | 229 | 0.7 | 30-Sep 15:32 | 30-Sep 16:14 |
| Deploy 03SGP-1B | 52° | 50.776' | N | 169° | 27.271' | W | 0.0 | 10 | 0.0 | 229 | 0.7 | 30-Sep 16:14 | 30-Sep 16:56 |
| CTD/nutrients at 03SGP-1B | 52° | 50.776' | N | 169° | 27.271' | W | 0.0 | 10 | 0.0 | 229 | 0.4 | 30-Sep 16:56 | 30-Sep 17:20 |
| CTD at 03SG-4A | 52° | 41.361' | N | 169° | 34.486' | W | 10.4 | 11.5 | 0.9 | 111 | 0.4 | 30-Sep 18:14 | 30-Sep 18:38 |
| Recover 03SG-4A | 52° | 41.361' | N | 169° | 34.486' | W | 0.0 | 10 | 0.0 | 111 | 0.7 | 30-Sep 18:38 | 30-Sep 19:20 |
| Deploy 03SG-4B | 52° | 41.361' | N | 169° | 34.486' | W | 0.0 | 10 | 0.0 | 111 | 0.7 | 30-Sep 19:20 | 30-Sep 20:02 |
| CTD at 03SG-4B | 52° | 41.361' | N | 169° | 34.486' | W | 0.0 | 10 | 0.0 | 111 | 0.4 | 30-Sep 20:02 | 30-Sep 20:26 |
| CTD at 03SG-5A | 52° | 43.991' | N | 169° | 23.642' | W | 7.1 | 11.5 | 0.6 | 112 | 0.4 | 30-Sep 21:03 | 30-Sep 21:27 |
| Recover 03SG-5A | 52° | 43.991' | N | 169° | 23.642' | W | 0.0 | 10 | 0.0 | 112 | 0.7 | 30-Sep 21:27 | 30-Sep 22:09 |
| Deploy 03SG-5B | 52° | 43.991' | N | 169° | 23.642' | W | 0.0 | 10 | 0.0 | 112 | 0.7 | 30-Sep 22:09 | 30-Sep 22:51 |
| CTD at 03SG-5B | 52° | 43.991' | N | 169° | 23.642' | W | 0.0 | 10 | 0.0 | 112 | 0.4 | 30-Sep 22:51 | 30-Sep 23:15 |
| Recover 03AMP-1A | 52° | 26.138' | N | 171° | 27.312' | W | 77.2 | 10 | 7.7 | 426 | 2.0 | 01-Oct 06:58 | 01-Oct 08:58 |
| Recover 03AMP-2A | 52° | 24.995' | N | 171° | 39.963' | W | 7.8 | 10 | 0.8 | 459 | 2.0 | 01-Oct 09:45 | 01-Oct 11:45 |
| Recover 03AMP-3A | 52° | 23.991' | N | 171° | 54.995' | W | 9.2 | 10 | 0.9 | 308 | 2.0 | 01-Oct 12:40 | 01-Oct 14:40 |
| Recover 03AMP-4A | 52° | 22.971' | N | 172° | 07.199' | W | 7.5 | 10 | 0.8 | 362 | 2.0 | 01-Oct 15:25 | 01-Oct 17:25 |
| Deploy 03AMP-4B | 52° | 22.971' | N | 172° | 07.199' | W | 0.0 | 10 | 0.0 | 362 | 2.0 | 01-Oct 17:25 | 01-Oct 19:25 |
| Deploy 03AMP-3B | 52° | 23.991' | N | 171° | 54.995' | W | 7.5 | 10 | 0.8 | 308 | 2.0 | 01-Oct 20:10 | 01-Oct 22:10 |
| Deploy 03AMP-2B | 52° | 24.995' | N | 171° | 39.963' | W | 9.2 | 10 | 0.9 | 459 | 2.0 | 01-Oct 23:06 | 02-Oct 01:06 |
| Deploy 03AMP-1B | 52° | 26.138' | N | 171° | 27.312' | W | 7.8 | 10 | 0.8 | 426 | 2.0 | 02-Oct 01:53 | 02-Oct 03:53 |
| CTD at 03AMP-1B | 52° | 26.138' | N | 171° | 27.312' | W | 0.0 | 10 | 0.0 | 426 | 0.7 | 02-Oct 03:53 | 02-Oct 04:33 |
| CTD at 03AMP-2B | 52° | 24.995' | N | 171° | 39.963' | W | 7.8 | 10 | 0.8 | 459 | 0.7 | 02-Oct 05:20 | 02-Oct 06:01 |
| CTD at 03AMP-3B | 52° | 23.991' | N | 171° | 54.995' | W | 9.2 | 10 | 0.9 | 308 | 0.6 | 02-Oct 06:57 | 02-Oct 07:32 |
| CTD at 03AMP-4B | 52° | 22.971' | N | 172° | 07.199' | W | 7.5 | 10 | 0.8 | 362 | 0.6 | 02-Oct 08:17 | 02-Oct 08:54 |
| Arrive Dutch Harbor, Alaska | 53° | 54.500' | N | 166° | 30.900' | W | 221.4 | 10 | 22.6 | 112 | 3.0 | 03-Oct 07:30 | 03-Oct 10:30 |
| Drag for FATE mooring | 58° | 15.612' | N | 147° | 41.214' | W | 679.4 | 11 | 61.8 | 112 | 24.0 | 06-Oct 00:16 | 07-Oct 00:16 |
| Arrive Seattle, Washington | 47° | 39.000' | N | 122° | 18.000' | W | 1106.1 | 11 | 100.6 | 112 | 0.0 | 11-Oct 04:49 |  |

  - Cruise MF-03-12 Mooring Sites