DRAFT Cruise Instructions

ECO-FOCI

NOAA Ship Oscar Dyson, Cruise DY-10-03

Apr 23 – May 3, 2010

Chief Scientist – Carol DeWitt, NOAA/PMEL

- DRAFT CRUISE INSTRUCTIONS

  - Cruise Title – Ecosystem and Fisheries-Oceanography Coordinated Investigations (Eco-FOCI).  Bering Sea Moorings.

  - Cruise Numbers: DY-10-04

    - Cruise Number – DY-10-03

    - Eco-FOCI Number – 1-DY-10

  - Cruise Dates: 23 Apr – 3 May, 2010

    - Departure – Kodiak, AK - 23 Apr, 1500 L

    - Arrival – Dutch Harbor, AK - 3 May, 0900 L

  - Operating Area – Bering Sea

- CRUISE OVERVIEW

  - Cruise Objectives – The primary objective of this cruise will be mooring operations – including recoveries, deployments and searches - in the Bering Sea.  Depending on ice conditions, the following mooring operations will be conducted on this cruise:

| OPERATIONS | SITE | LATITUDE | LONGITUDE |
| --- | --- | --- | --- |
| Recover/Deploy | Pavlof Bay | 55° 10.87' N | 161° 41.15' W |
| Recover/Deploy | Bristol Bay 2 | 56° 29.84' N | 160° 59.66' W |
| Recover/Deploy | Mooring 2 | 56° 51.23' N | 164° 04.54' W |
| Recover/Deploy | Mooring 4 | 57° 50.78' N | 168° 51.65' W |
| Recover/Deploy | Mooring 5 | 59° 55.00' N | 171° 42.24' W |

  - Applicability - These instructions, with FOCI Standard Operating Instructions for NOAA Ship Oscar Dyson, dated November 11, 2005, present complete information for this cruise.

  - Participating Organizations

NOAA - Pacific Marine Environmental Laboratory (PMEL)

7600 Sand Point Way N.E., Seattle, Washington  98115-6439

  - Personnel

    - Chief Scientist

| Name | Gender | # Nationality | Affiliation | E-mail Address |
| --- | --- | --- | --- | --- |
| Carol DeWitt | F | USA | PMEL | Carol.DeWitt@noaa.gov |

Other Participating Scientists –

| Name | Gender | Nationality | Affiliation | E-mail Address |
| --- | --- | --- | --- | --- |
| William Floering Scott McKeever Eric Wisegarver David Strausz Miriam Doyle Morgan Busby Jennifer Miksis-Olds TBA TBA | M M M M F M F | USA USA USA USA USA USA USA USA USA | PMEL PMEL PMEL PMEL AFSC AFSC Penn State FWS UAF | William.Floering@noaa.gov Scott.McKeever@noaa.gov Eric.Wisegarver@noaa.gov David.Strausz@noaa.gov Miriam.Doyle@noaa.gov Morgan.Busby@noaa.gov jlm91@psu.edu |

    - Foreign Nationals   None

  - Administration

    - Ship Operations

Marine Operations Center, Pacific

1801 Fairview Avenue East, Seattle, Washington 98102-3767

Telephone: (206) 553-4548

Fax: (206) 553-1109

CDR Mike Francisco

Chief, Operations Division, Pacific (MOP1)

Telephone: (206) 553-8705

Cellular: (206) 390-7527/(206) 518-1941

E-mail: chiefops.MOP@noaa.gov

    - Scientific Operations

Dr. Phyllis J. Stabeno, PMEL		Dr. Jeffrey Napp, AFSC

Telephone: (206) 526-6453			Telephone: (206) 526-4148

E-mail: Phyllis.Stabeno@noaa.gov		E-mail: Jeff.Napp@noaa.gov

- OPERATIONS

  - Scientific Computer System (SCS) - The ship's SCS shall operate throughout the cruise, acquiring and logging data from navigation, meteorological, oceanographic, and fisheries sensors.  See FOCI Standard Operating Instructions for NOAA Ship Oscar Dyson (SOI 5.2) for specific requirements.

  - Staging Plan –    A container will be shipped to Kodiak prior to the cruise. The scientific party will be responsible for arranging vehicles for moving their equipment from the airport and/or docks.

  - De-staging Plan – The equipment will be off-loaded in Dutch Harbor and barged to Seattle.  The scientific party will be responsible for arranging vehicles for moving their equipment from the docks.

  - Cruise Plan – Due to the time of the year that this cruise occurs, the amount of mooring work accomplished and the order of operations will be highly dependent on ice, weather and daylight conditions.  Based on the past several years, it is unlikely that we will be able to reach Bering Sea site 5.  And although we may reach Bering Sea site 4, the current and predicted future ice conditions may prevent us from reaching all of the four surrounding CTD sites.

The ship will depart on Friday, April 23, 2010.

- Pavlof Bay – Prior to mooring operations, a calibration CTD will be completed. Mooring operations will consist of recovering one subsurface mooring and deploying one subsurface mooring.  No CTD will be required after the mooring deployment.

- Drifter deployments – At each of five locations, there will be two drifter deployments.  One of the two drifters will be drogued at 20 m and the other drifter will be drogued at 40 m.

- Bristol Bay – A mooring – 07KC-1A – was deployed at the inshore site in April 2007.  It wa not recovered in 2008 due to cruise time limitations.  It is likely that the release on this mooring will be heavily encrusted with marine growth and will probably not release on its own.  In the past we have "snagged" the mooring with drag hooks using the aft facing non conductive wire winch.  PMEL will provide the drag gear. No mooring will be deployed at this inshore site.  We will then proceed to the offshore Bristol Bay site.  Prior to these mooring operations, a calibration CTD will be completed.  Mooring operations will consist of recovering one subsurface mooring and deploying one subsurface mooring.  No CTD will be required after the mooring deployment.

- FOCI Bering Sea Site 2 – Depending on arrival timing, we will either proceed with mooring operations or with the CTD “box”.  Prior to mooring operations, calibration CTDs (with nutrient and chlorophyll samples) will be completed.  Mooring operations will consist of recovering two subsurface moorings and deploying one surface and two subsurface moorings.  After the completion of all mooring operations, a CTD, with nutrient and chlorophyll samples, a MARMAP Bongo tow with 20 and 60 cm bongos and triplicate CalVET tows will be completed approximately 0.5 mile from the mooring site.  At the four stations surrounding Site 2, a CTD and 20/60 bongo tow will be completed.  Depending on weather and daylight conditions, a mooring search for a subsurface ADCP mooring (08BSP-2A) will be conducted either before or after the mooring recovery/deployment operations.  The mooring we will be searching for was deployed during a DYSON cruise in May 2008.  In September 2008, the mooring did not respond and an 18 hr search was conducted.  A search plan – that takes into consideration the previous search – will be provided to the ship in the draft cruise instructions.

- FOCI Bering Sea Site 4 – If ice conditions allow, the ship will transit from FOCI Bering Sea Site 2 to FOCI Bering Sea Site 4.  Depending on arrival timing, we will either proceed with mooring operations or with the CTD “box”.  Prior to mooring operations, a calibration CTD (with nutrient and chlorophyll samples) will be completed.  Mooring operations will consist of recovering three subsurface moorings and deploying two subsurface moorings.  After the completion of all mooring operations, a CTD, with nutrient and chlorophyll samples, a MARMAP Bongo tow  with 20 and 60 cm bongos and triplicate CalVET tows will be completed approximately 0.5 mile from the mooring site.  At the four stations surrounding Site 2, a CTD and 20/60 bongo tow will be completed.

- FOCI Bering Sea Site 5 – If ice conditions and time allow, the ship will transit from FOCI Bering Sea Site 4 to FOCI Bering Sea Site 5.  Depending on arrival timing, we will either proceed with mooring operations or with the CTD “box”.  Prior to mooring operations, a calibration CTD (with nutrient and chlorophyll samples), a MARMAP Bongo tow with 20 and 60 cm bongos and triplicate CalVET tows will be completed.  Mooring operations will consist of recovering and redeploying two subsurface moorings.  After the completion of all mooring operations, a CTD will be completed.  A CTD, with nutrient and chlorophyll samples and a 20/60 Bongo tow will be completed at the four stations surrounding Site 5.

- Cape Newenham CTDs – If time allows, we will complete a CTD transect along the Cape Newenham line.

  - Station Locations – See Table 1.

  - Station Operations - The following are operations to be conducted on this cruise.  The procedures for these operations are listed in the FOCI Standard Operating Instructions for NOAA Ship Oscar Dyson (SOI).  Operations not addressed in the SOI and changes to standard procedures are addressed below.

- CTD/Water Sample Operations (SOI 3.2.1)

- Chlorophyll and Nutrient Sampling Operations (SOI 3.2.10)

- ARGOS Satellite-Tracked Drifter Buoy Deployments (SOI 3.2.11)

- MARMAP Bongo Tows (SOI 3.2.2)

- Dissolved Oxygen Sampling

- SIMRAD EK-60 and 12 Khz Simrad ES-60 Scientific Echosounder Monitoring (SOI 3.2.12)

- Simrad ME-70 Downward-Facing Multi-Beam Sonar

- Recovery and deployment of surface and subsurface moorings

  - Underway Operations - The following are underway operations to be conducted on this cruise.  The procedures for these operations are listed in the FOCI Standard Operating Instructions for NOAA Ship Oscar Dyson(SOI).  Operations not addressed in the SOI and changes to standard procedures are addressed below.

- Acoustic Doppler Current Profiler (ADCP) Operations (SOI 3.2.14.1)

- Scientific Computer System (SCS) data acquisition (SOI 3.2.14.2)

- Fluorometer monitoring (SOI 5.3)

- Thermosalinograph monitoring (SOI 5.3)

  - Applicable Restrictions - None

  - Small Boat Operations – Small boat operations may be requested to assist with mooring operations

- FACILITIES

  - Equipment and Capabilities Provided by Ship

- Oceanographic winch with slip rings and 3-conductor cable terminated for CTD

- 12 Khz hull mounted Edgetech Acoustic release transducer

- Manual wire-angle indicator

- Oceanographic winch with slip rings and 3-conductor cable terminated for the SBE SEACAT for net tow operations

- Sea-Bird Electronics’ SBE 911plus CTD system with stand each CTD system should include underwater CTD weights and pinger.  There should be a deck unit for the system

- Sea-Bird Electronics’ SBE-19 SEACAT system

- 10-liter Niskin sampling bottles for use with rosette (10 plus 4 spares)

- Conductivity and temperature sensor package to provide dual sensors on the CTD (primary)

- Wire speed indicators and readout for oceo winches

- For meteorological observations: 2 anemometers (one R. M. Young system interfaced to the SCS) calibrated air thermometer (wet-and dry-bulb) and a calibrated barometer and/or barograph

- Freezer space for storage of  chemical samples (both blast and storage freezers, -20° C and -80° C) turned on and operating

- SIMRAD ES-60 and EK-60 echosounder

- Simrad ME-70 Downward-Facing Multi-Beam Sonar

- RD Instruments’ ADCP written to CD

- Use of PCs in laboratories for data analysis

- Scientific Computer System (SCS)

- Minimum of 2 computers with internet and e-mail access

- Removable stern platform (in place)

- Laboratory space with storage space

- Seawater hoses and nozzles to wash nets

- Aft leading non-conductive wire winch

- Adequate deck lighting for night-time operations

- Navigational equipment including GPS and radar

- Safety harnesses for working on quarterdeck and fantail

- Ship’s crane(s) used for loading and/or deploying

  - Equipment and Capabilities Provided by Scientists

- Fluorometer, light meter and dual oxygen sensors to be mounted on CTD

- Conductivity and temperature sensor package to provide dual sensors on the CTD (backup)

- Sea-Bird Electronics’ SBE-19 SEACAT system (backup)

- 20 cm and 60 cm Bongo sampling arrays

- Manual wire indicator

- IAPSO standard water

- Surface mooring (FOCI biophysical platform)

- Subsurface moorings

- Miscellaneous scientific sampling and processing equipment

- Chlorophyll and nutrient sampling equipment

- Satellite-tracked drifters, 10

- DISPOSITION OF DATA AND REPORTS

  - The following data products will be included in the cruise data package:

- NOAA Form 77-13d - Deck Log - Weather Observation Sheets

- Electronic Marine Operations Abstracts

- SCS backup

- Calibration Sheets for all ship's instruments used

- CTD Cast Information/Rosette Log

- ADCP Log Sheets

- ADCP CD (CD-RW)

- Ultra-cold Freezer Temperature Daily Log (SOI 5.4)

  - Pre- and Post-cruise Meetings - Cruise meetings may be held in accordance with FOCI Standard Operating Instructions for NOAA Ship Oscar Dyson (SOI 5.5).

- ADDITIONAL PROJECTS

  - Definition - Ancillary and piggyback projects are secondary to the objectives of the cruise and should be treated as additional investigations.  The difference between the two types of secondary projects is that an ancillary project does not have representation aboard and is accomplished by the ship's force.

  - Ancillary Projects - Any ancillary work done during this project will be accomplished with the concurrence of the Chief Scientist and on a not-to-interfere basis with the programs described in these instructions and in accordance with the NOAA Fleet Standing Ancillary Instructions.

  - Piggyback Projects - None

- HAZARDOUS MATERIALS The field party chief shall be responsible for complying with MOCDOC 15, Fleet Environmental Compliance #07, Hazardous Material and Hazardous Waste Management Requirements of Visiting Scientists.  July 2002

  - Inventory

Haz-mat inventory to be provided on next revision of DRAFT instructions.

  - Material Safety Data Sheet (MSDS)

(Provide an electronic MSDS for each hazardous material that will be brought aboard the ship.  These may be included as part of the Appendix of these instructions.  If so, refer here to that section of the Appendix.  Otherwise, state that MSDSs will be forwarded separately.)

- MISCELLANEOUS

  - Communications - Specific information on how to contact the NOAA Ship Oscar Dyson and all other fleet vessels can be found at:

http://www.moc.noaa.gov/phone.htm

  - Important Telephone and Facsimile Numbers and E-mail Addresses

    - Pacific Marine Environmental Laboratory (PMEL):

FOCI - Ocean Environmental Research Division (OERD2):

- (206) 526-4700 (voice)

- (206) 526-6485 (fax)

Administration:

- (206) 526-6810 (voice)

- (206) 526-6815 (fax)

E-Mail: FirstName.LastName@noaa.gov

    - Alaska Fisheries Science Center (AFSC):

FOCI - Resource Assessment and Conservation Engineering (RACE):

- (206) 526-4171 (voice)

- (206) 526-6723 (fax)

E-Mail: FirstName.LastName@noaa.gov

    - NOAA Ship Oscar Dyson - Telephone methods listed in order of increasing expense:

United States Coast Guard - Kodiak, Alaska

- (907) 486-0553   USCG Operations

Cellular:

- CO 206-403-8433

- XO 206-295-0775

- OPS/OOD 206-295-0550

Iridium:

- 808-659-0050

INMARSAT B

- 011-872-336-995-921 ( fax)

- 011-872-336-995-920 (voice)

- Dutch Harbor Cell CO 907-359-1801

- Dutch Harbor Cell XO 907-359-1802

    - Marine Operations Center, Pacific (MOP):

Operations Division (MOP1)

- (206) 553-4548 (voice)

- (206) 553-1109 (facsimile)

E-Mail: FirstName.LastName@noaa.gov

E-Mail to Radio Room: Radio.Room@noaa.gov

  - Foreign National Access and Deemed Export Controls on NMAO Vessels

None

- APPENDICES

  - Equipment Inventory

One surface mooring donut, bridle and tower - 1,500 lbs - 8ft wide 16 ft tall.

One anchor for surface moorings - 4,500 lbs - 4ft x 3 ft

Surface mooring chain 4,500 lbs

Subsurface mooring anchors (railroad wheels):

1 X 740 lbs

5 X 1,600 lbs

1 X 2,100 lbs

300 Khz ADCP in syn. Foam float - 600 lbs x 3

Tapps instrument package in syntactic foam float - 1,000 lbs

Steel floats for subsurface moorings - 3,000 lbs

Acoustic releases 150 lbs x 9 or 10

Aurals - 100 lbs x 3

Rain gauge - 50 lbs x 1 or 2

AWCP - 300 lbs x 2

Misc instruments and cages   1,500 lbs

Equipment footlockers, shackles, chain - 200 lbs x 2

Mooring chain on spools - 400 lbs x 3

Grapple hooks and chains - 300 lbs

Drifters – 60 lbs x 13

Bongo frames, nets, CalVET, and weight - 150 lbs

  - HAZMAT Inventory

8.1.1 Hazmat Inventory, DeWitt:

| Chemical | CAS Number | Respondee | Org | Qty | H | F | R | Storage Color Code | Hazard Class | Packing Group Number | UN # | Response Indices |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Battery, Lithium | mixture | DeWitt | PMEL | * | 2 | 2 | 3 | General | 9 | II | 3090 |  |
| Tributyltin Oxide | 56-35-9 | DeWitt | PMEL | 6 oz. | 3 | 1 | 0 | Poison | N. R. |  |  | 1 |
| Battery, Lithium Tadiran | mixture | Strausz | PMEL | 384 | 1 | 1 | 2 | General | 9 | II | 3090 |  |
| Formaldehyde, 37% | 50-00-0 | Busby | AFSC | 3, 20- L | 3 | 2 | 2 | ### Flammable | 3 & 8 | III | 1198 | 1 |
| Sodium Borate Solution, Saturated | mix | Busby | AFSC | 20- L | 1 | 0 | 0 | General | Not regulated | N/A |  | 2 |
| Ethyl Alcohol, 95% | UN1170 | Busby | AFSC | 2, 1 gal | 2 | 3 | 1 | Flammable | 3 | II, III | 1993 | 1 |
| Ethyl Alcohol, 100%, genetics grade | UN1170 | Busby | AFSC | 2, 1 gal | 3 | 3 | 1 | Flammable | 3 | II, III | 1987 | 1 |
| Ethylene glycol | 107-21-1 | Busby | AFSC | 250 ml | 2 | 1 | 1 | General | Not regulated | N/A |  | 2 |
| Spill Response 1: Stop the leak, if possible. Ventilate the space involved. Absorb, sweep up, and place in container for disposal. Shut off or remove all ignition sources. Prevent waterway contamination. Construct a dike to prevent spreading. Collect run-off (water) and transfer to drums or tanks for later disposal. |  |  |  |  |  |  |  |  |  |  |  |  |

* Lithium and Alkaline battery quantities as follows:

34 - SBE-39 (9-V lithium battery)

10 - Microcat (6 lithium battery sticks and anti-fouling on conductivity cells)

7 - ECO-fluorometer (6 9-V lithium batteries)

6 - Seacat (9 D alkaline cells and antifouling cylinders on conductivity cells)

384 – Lithium Tadiran TL-5930 “D” cell sized batteries

(In 12 32 cell battery packs)

6 – Alkaline Battery Packs (126 D cell alkaline batteries in each pack) (AWCP)

48 – Hybrid Layer Capacitors (4 in each 32 cell battery pack)

Spare batteries, in a hazmat can:

6 9-V lithium batteries

12 9-V alkaline batteries

12 lithium battery sticks

2 packs of anti-fouling cylinders

  - Figures

| Area | Activity | Latitude |  |  | Longitude |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pavlof Bay | CTD at 09PA-1A | 55° | 10.87 | N | 161° | 41.15 | W |
|  | Recover 09PA-1A | 55° | 10.87 | N | 161° | 41.15 | W |
|  | Deploy 10PA-1A | 55° | 10.87 | N | 161° | 41.15 | W |
| Bering Sea | Drifter deployment, 40 m | 55° | 57.31 | N | 165° | 4.81 | W |
|  | Drifter deployment, 20 m | 55° | 57.69 | N | 165° | 4.13 | W |
|  | Drifter deployment, 40 m | 56° | 2.69 | N | 165° | 4.82 | W |
|  | Drifter deployment, 20 m | 56° | 3.07 | N | 165° | 4.14 | W |
|  | Drifter deployment, 20 m | 56° | 3.09 | N | 164° | 54.50 | W |
|  | Drifter deployment, 40 m | 56° | 2.69 | N | 164° | 55.18 | W |
|  | Drifter deployment, 20 m | 56° | 0.38 | N | 164° | 59.32 | W |
|  | Drifter deployment, 40 m | 56° | 0.00 | N | 165° | 0.00 | W |
|  | Drifter deployment, 20 m | 55° | 57.69 | N | 164° | 54.51 | W |
|  | Drifter deployment, 40 m | 55° | 57.31 | N | 164° | 55.19 | W |
| Bristol Bay | Search for 07KC-1A | 56° | 25.61 | N | 160° | 13.12 | W |
|  | CTD at 09KC-2A | 56° | 29.84 | N | 160° | 59.66 | W |
|  | Recover 09KC-2A | 56° | 29.84 | N | 160° | 59.66 | W |
|  | Deploy 10KC-2A | 56° | 30.00 | N | 161° | 0.00 | W |
| BS Site 2 | CTDs (2) at site 2 chlorophylls and nutrients 0.5 mi away fm mrg site | 56° | 52.500' | N | 164° | 03.000' | W |
|  | Recover 09BS-2C | 56° | 51.232' | N | 164° | 04.537' | W |
|  | Recover 09BSP-2B | 56° | 51.570' | N | 164° | 03.801' | W |
|  | Deploy 10BSM-2A | 56° | 52.000' | N | 164° | 03.000' | W |
|  | Deploy 10BSP-2A | 56° | 52.000' | N | 164° | 02.000' | W |
|  | Deploy 10BST-2A | 56° | 52.000' | N | 164° | 04.000' | W |
|  | 20/60 cm bongo (150/333 nets) - site 2 | 56° | 52.500' | N | 164° | 03.000' | W |
|  | CalVET (triplicate) - site 2 | 56° | 52.500' | N | 164° | 03.000' | W |
|  | CTDs (3) at site 2 chlorophylls and nutrients 0.5 mi away fm mrg site | 56° | 52.500' | N | 164° | 03.000' | W |
|  | CTD - site 2/south chlorophylls | 56 | 40.00 | N | 163 | 52.00 | W |
|  | 20/60 cm bongo (150/333 nets) - site 2/south | 56 | 40.00 | N | 163 | 52.00 | W |
| Area | Activity | Latitude |  |  | Longitude |  |  |
|  | 20/60 cm bongo (150/333 nets) - site 2/east | 56 | 56.50 | N | 163 | 50.01 | W |
|  | CTD - site 2/east chlorophylls | 56 | 56.50 | N | 163 | 50.01 | W |
|  | CTD - site 2/north chlorophylls | 57 | 01.00 | N | 164 | 13.00 | W |
|  | 20/60 cm bongo (150/333 nets) - site 2/north | 57 | 01.00 | N | 164 | 13.00 | W |
|  | 20/60 cm bongo (150/333 nets) - site 2/west | 56 | 46.00 | N | 164 | 20.00 | W |
|  | CTD - site 2/west chlorophylls | 56 | 46.00 | N | 164 | 20.00 | W |
|  | Search for 08BSP-2A | 56° | 51.928' | N | 164° | 03.185' | W |
| BS Site 4 | CTDs (2) at site 4 chlorophylls and nutrients 0.5 mi away fm mrg site | 57° | 52.200' | N | 168° | 53.000' | W |
|  | Recover 09BS-4B | 57° | 50.777' | N | 168° | 51.652' | W |
|  | Recover 09BSP-4B | 57° | 50.966' | N | 168° | 51.970' | W |
|  | Recover 09BSIP-4B | 57° | 50.701' | N | 168° | 52.112' | W |
|  | Deploy 10BSP-4A | 57° | 51.210' | N | 168° | 52.211' | W |
|  | Deploy 10BS-4A | 57° | 51.210' | N | 168° | 52.211' | W |
|  | CTDs (2) at site 4 chlorophylls and nutrients 0.5 mi away fm mrg site | 57° | 51.500' | N | 168° | 53.000' | W |
|  | 20/60 cm bongo (150/333 nets) - site 4 | 57° | 51.500' | N | 168° | 53.000' | W |
|  | CalVET (triplicate) - site 4 | 57° | 51.500' | N | 168° | 53.000' | W |
|  | 20/60 bongo - site 4 south | 57° | 39.200 | N | 169° | 1.200 | W |
|  | CTD - site 4 south chlorophylls | 57° | 39.200 | N | 169° | 1.200 | W |
|  | 20/60 bongo - site 4 east | 57° | 46.000 | N | 168° | 28.000 | W |
|  | CTD - site 4 east chlorophylls | 57° | 46.000 | N | 168° | 28.000 | W |
|  | CTD - site 4 north chlorophylls | 58° | 4.000 | N | 168° | 43.800 | W |
|  | 20/60 bongo - site 4 north | 58° | 4.000 | N | 168° | 43.800 | W |
|  | CTD - site 4 west chlorophylls | 57° | 55.600 | N | 169° | 19.300 | W |
|  | 20/60 bongo - site 4 west | 57° | 55.600 | N | 169° | 19.300 | W |
| BS Site 5 | 20/60 bongo - site 5 south | 59° | 42.000 | N | 171° | 30.000 | W |
|  | CTD - site 5 south chlorophylls | 59° | 42.000 | N | 171° | 30.000 | W |
| Area | Activity | Latitude |  |  | Longitude |  |  |
|  | 20/60 bongo - site 5 east | 59° | 53.880 | N | 171° | 15.500 | W |
|  | CTD - site 5 east chlorophylls | 59° | 53.880 | N | 171° | 15.500 | W |
|  | CTD - site 5 north chlorophylls | 60° | 4.500 | N | 172° | 0.000 | W |
|  | 20/60 bongo - site 5 north | 60° | 4.500 | N | 172° | 0.000 | W |
|  | CTD - site 5 west chlorophylls | 59° | 53.880 | N | 172° | 10.000 | W |
|  | 20/60 bongo - site 5 west | 59° | 53.880 | N | 172° | 10.000 | W |
|  | CTDs (2) at site 5chlorophylls and nutrients0.5 mi away fm mrg site | 59° | 54.998' | N | 171° | 42.244' | W |
|  | Recover 09BS-5B | 59° | 54.998' | N | 171° | 42.244' | W |
|  | Recover 09BSP-5B | 59° | 54.595' | N | 171° | 42.500' | W |
|  | Deploy 10BSP-5A | 59° | 54.595' | N | 171° | 42.500' | W |
|  | Deploy 10BS-5A | 59° | 54.998' | N | 171° | 42.244' | W |
|  | CTDs (2) at site 5 chlorophylls and nutrients 0.5 mi away fm mrg site | 59° | 54.998' | N | 171° | 42.244' | W |
|  | 20/60 cm bongo (150/333 nets) - site 5 | 59° | 54.998' | N | 171° | 42.244' | W |
|  | CalVET (triplicate) - site 5 | 59° | 54.998' | N | 171° | 42.244' | W |
| Newenham | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 57° | 0.00 | N | 163° | 54.00 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 56° | 53.92 | N | 164° | 2.29 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 56° | 42.28 | N | 164° | 30.53 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 56° | 33.81 | N | 164° | 54.27 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 56° | 25.30 | N | 165° | 18.17 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 56° | 16.67 | N | 165° | 42.04 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 56° | 8.24 | N | 166° | 6.21 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 55° | 59.23 | N | 166° | 30.62 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 55° | 51.04 | N | 166° | 54.43 | W |
| Area | Activity | Latitude |  |  | Longitude |  |  |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 55° | 41.83 | N | 167° | 18.22 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 55° | 33.37 | N | 167° | 42.06 | W |
|  | CTD - Cape Newenham (chlor at: 0, 10,20,30,40,50 m) | 55° | 26.06 | N | 168° | 4.31 | W |